Faculty of Engineering                                                                                                 CEE216/CEE210

Electrical Engineering Dep                                                                                               year Communication

Instructor// Dr: Gihan Naguib                                                                                       Spring 2020

TA: Muhammad Dakheel

Simulation and Designing 32-bit Mips processor

Group members: -

1.عبدالله محمد أمين صادق (2820)

2.علاء محمد صديق محمد (2821)

3.جاسر جمال أحمد أحمد (2810)

Group number (9)

Introduction

It’s so clear to any engineer studies computer architecture that there is some mane steps in designing a processor specially single cycle processor and we’ll conclude them roughly as following below.

The steps of design a single cycle processor: -

1) There are 5 main components -functions- in single cycle processor

1. PC                 2. Register file         3. ALU         4. Memory which is helpful to -be separated into Instruction_Mem and Data_Mem       5. Control unit

.                           (علاء محمد + عبدالله محمد + جاسر جمال)

2) ALU implementation which is made to execute all the required operations        given in the project               (عبدالله محمد + جاسر جمال).

3) Register file implementation      (جاسر جمال)

4)  ALU Data Path.                            (عبدالله محمد ).

5)Data path j-type instructions       (علاء محمد).

6)Connections of the circuit (علاء محمد + عبدالله محمد + جاسر جمال ).

7) Control signals table        (علاء محمد + جاسر جمال ).

8) Control unit                       (عبدالله محمد + علاء محمد )

9) converting test code to machine language then entering instructions to memory                                   (عبدالله محمد+ جاسر جمال ).

10) Repairing problems and errors in the design                                                   (عبدالله محمد + علاء محمد)   .

11) Following the Execution of  the program and evaluating the final values           .                              (عبد الله محمد + علاء محمد + جاسر جمال).

(12Making the documentation and the report ( جاسر جمال)

- Design and Implementation: -

Firstly, we’ll talk about a useful component in performing most of our instructions called Arithmetic and Logical Unit, simply (ALU) which is considered below:

- Logical operations:

AND, OR, XOR, EQV -XNOR-.

- Arithmetic operation:

add, subtract _ with detecting over flow case.

- Comparison operation:

beq, bne, SLT, BEQZ, BNEZ, BLTZ, BGEZ, BGTZ, BLEZ.

- Shifting operation:

sll, srl, RoR.

ALU has 2 outputs:

.1the result of “logical, arithmetic, shifting and                                                      Comparison operation”

.2A flag for overflow

Implement a 32-bit ALU to perform all the required operations-:

Secondly, we’ll talk about program counter or simply (PC) that is used to fetch instruction from memory -which is separated here to instruction _mem and data_mem- and hold the address of the next instruction for execution.

- We have modified our (PC) to support the required given instructions as it’s considered in the following figure: -

Implement a program counter (PC): -

Thirdly, we’ll talk about how we can store _ usage data in our processor specially, I mean register file that is an array of a processor, but differs from memory as it ’s so small comparing with memory, but too fast.

- There is two useful ways to implement a Register file are considered in below: -

1. Using Mux.

2.Using a decoder and tri-state.

- We have used the second method as it has two advantages: -

1.Reducing cost                                            2. Power consumption

- As we only have need decoders and tri-state which are less hardware components comparing with using as we know from logic design Muxs consists of AND, OR, NOT logic gates.

Implement a Register file containing eight 32-bit registers: -

Next here we’ll talk about building a data-path that connects our devices -components – together- that we’ve talked about, earlier- to perform a certain instruction after knowing its type by the help of control unit which we’ll talk about at the end.

- We’ve applied connections which support the four given types of instructions R-type, I-type, B-type, J-type.

Building a data-path: -

Finally, we’ll talk about the mind of the brain. I mean control unit that has a big role in generating signals to control our data-path and enabling components related to a certain instruction.

- We take inputs(I/o , MR ,WR ) from processor inter-face and fed it to a decoder to give us op-codes which is fed to our main control  as a special marque which differs from types of instruction .However for arithmetic operation  add , sub , addI , ….etc or specially, R-type instructions op-code doesn’t enough we should have another special marque with it which is Func (2bits) that all is  fed to (ALU) through  [ ALU -op ] which is a subset of op-code.

Controlling B-type and J-type instructions: -

The full main control circuit: -

The full main control circuit: -

Table giving the control signal values for each instruction: -

Table giving the control signal values for each instruction: -

The logic equations for each control signal: -

| Groups |  |
| --- | --- |
| (20,21,22,23,28,29,30,31) | A.C |
| (24,25,26,27,28,29,30,31) | A.B |

R1 = AC + AB

| Groups |  |
| --- | --- |
| (2,3,6,7,10,11,14,15,18,19,22,23,26,27,30,31) | D |
| (16,17,18,19,20,21,22,23,24,25,26,27,28,29,30,31) | A |

R2 = D + A

| Groups |  |
| --- | --- |
| (18,19,22,23,26,27,30,31) | A.D |

W (Reg_dis) = AD

| Groups |  |
| --- | --- |
| (20,21,22,23,28,29,30,31) | A.C |
| (24,25,26,27,28,29,30,31) | A.B |

Imm = AC + AB

| Groups |  |
| --- | --- |
| (16,17,18,19,20,21,22,23,24,25,26,27,28,29,30,31) | A |

Mem to Reg = A

| Groups |  |
| --- | --- |
| (27,31) | A.B.D.E |

R7 = ABDE

| Groups |  |
| --- | --- |
| (28) | A.B.C.D.E |

SET = ABCD'E'

| Groups |  |
| --- | --- |
| (29) | A.B.C.D.E |

SSET = ABCD'E

| Groups |  |
| --- | --- |
| (0,1,2,3,4,5,6,7) | A.B |
| (0,1,2,3,8,9,10,11) | A.C |
| (0,4,8,12) | A.D.E |
| (0,2,16,18) | B.C.E |
| (3,11,19,27) | C.D.E |
| (28,29,30,31) | A.B.C |

Reg_W = A'B' + A'C' + A'D'E' + B'C'E' + C'DE + ABC

| Groups |  |
| --- | --- |
| (16,18) | A.B.C.E |

Mem_R = AB'C'E'

| Groups |  |
| --- | --- |
| (17,19) | A.B.C.E |

Mem_w = AB'C'E

| Groups |  |
| --- | --- |
| (20,21,22,23) | A.B.C |
| (14,15) | A.B.C.D |
| (24,25) | A.B.C.D |

Branch = AB'C + A'BCD + ABC'D'

| Groups |  |
| --- | --- |
| (30,31) | A.B.C.D |

J = ABCD

| Groups |  |
| --- | --- |
| (18,19,26,27) | A.C.D |

JR = AC'D

| Groups |  |
| --- | --- |
| (16,17,18,19,20,21,22,23,24,25,26,27,28,29,30,31) | A |
| (2,3,6,7,18,19,22,23) | B.D |
| (4,5,12,13,20,21,28,29) | C.D |
| (8,9,10,11,24,25,26,27) | B.C |

ALU Src = A + B'D + CD' + BC'

ALU Op:-

| Groups |  |
| --- | --- |
| (20,21,22,23,28,29,30,31) | A.C |
| (24,25,26,27,28,29,30,31) | A.B |

Y1 = AC + AB

| Groups |  |
| --- | --- |
| (12,13,14,15,28,29,30,31) | B.C |
| (10,11,14,15) | A.B.D |
| (16,17,18,19) | A.B.C |

Y2 = BC + A'BD + AB'C'

| Groups |  |
| --- | --- |
| (16,17,18,19,24,25,26,27) | A.C |
| (6,7,14,15) | A.C.D |
| (8,9,24,25) | B.C.D |

Y3 = AC' + A'CD + BC'D'

| Groups |  |
| --- | --- |
| (18,19,22,23,26,27,30,31) | A.D |
| (4,5,12,13) | A.C.D |
| (8,9,12,13) | A.B.D |
| (16,17,18,19) | A.B.C |

Y4 = AD + A'CD' + A'BD' + AB'C'

| Groups |  |
| --- | --- |
| (1,3,5,7,9,11,13,15,17,19,21,23,25,27,29,31) | E |

Y5 = E

| Instruction Bin_num | Instruction | Hex |
| --- | --- | --- |
| 1110000101010101 | set $r1, 0x55 | e155 |
| 1110100100010010 | sset $r1, 0x12 | e912 |
| 1110100100010000 | sset $r1, 0x10 | e910 |
| 1110100100100000 | sset $r1, 0x20 | e920 |
| 1110100100000000 | sset $r1, 0x00 | e900 |
| 1110011000100010 | set $r6, 0x22 | e622 |
| 1110111000000110 | sset $r6, 0x06 | ee06 |
| 1110111000000010 | sset $r6, 0x02 | ee02 |
| 1110111000010001 | sset $r6, 0x11 | ee11 |
| 001111000111 | eqv $r3, $r6, $r1 | 3c7 |
| 100000000001 | sub $r0, $r0, $r0 | 801 |
| 1000000100000000 | lw $r1, 0($r0) | 8100 |
| 1000001000000001 | lw $r2, 1($r0) | 8201 |
| 1000001100000010 | lw $r3, 2($r0) | 8302 |
| 0100010010001010 | addi $r4, $r4, 10 | 448a |
| 110010010001 | sub $r4, $r4, $r4 | c91 |
| 110001010000 | add $r4, $r2, $r4 | c50 |
| 110101001110 | slt $r5, $r2, $r3 | d4e |
| 0111010100000011 | beq $r5, $r0, 2 | 7503 |
| 101000101000 | add $r2, $r1, r2 | a28 |
| 0111000000011100 | beq $r0, $r0, -5 | 701c |
| 1000110000000000 | sw $r4, 0($r0) | 8c00 |
| 1111100000000110 | jal func | f806 |
| 0110111011001000 | ror $r6, $r6, 8 | 6ec8 |
| 1000010000000001 | lw $r4, 1($r0) | 8401 |
| 1000010100000010 | lw $r5, 2($r0) | 8502 |
| 111011010100 | add $r6, $r6, $r5 | ed4 |
| 011100000001 | beq $r0, $r0, -1 | 701 |
| 1111 | sub $r0, $r0, $r0 | f |
| 100000000001 | lw $r1, 0($r0) | 801 |
| 1000000100000000 | lw $r2, 0($r1) | 8100 |
| 1000001000100000 | lw $r3, 1($r1) | 8220 |
| 1000001100100001 | and $r4, $r2, $r3 | 8321 |
| 010001001100 | or $r5, $r2, $r3 | 44c |
| 010101001101 | sw $r4, 1($r0) | 54d |
| 1000110000000001 | sw $r5, 2($r0) | 8c01 |
| 1000110100000010 | jr $r7 | 8d02 |

Ro= 0x00000000/0

R1= 0x00000000/0

R2= 0x00000000/0

R3= 0x00000000/0

R4= 0x00000000/0

R5= 0x00000000/0

R6= 0x00000011/17

R7= 0x00000000/0

| Memory address | Value |
| --- | --- |
| 00000 | 0x00000000/0 |